README File - G Code Controller for Windows

Version 1.14

Copyright 2005 Ability Systems Corporation

Read the Program License Agreement. Do not use the product if

you do not agree.

New Installation and

Upgrading from Version 1.1x

To install the G Code Controller files onto your hard

drive, run the Setup program provided on the distribution

diskette.

Place the distribution diskette in the A drive. From the Windows

95/98/ME/XP desktop, snap Start, then snap Run. Type A:\Setup

into the Open field, then snap Ok.

This software is keyed to function with the Indexer LPT Hardware

Assist Module (HAM) under a license file that must be copied to the

Indexer LPT directory. The name of the license file corresponds

to the serial number of the HAM. For example, if the serial

number of the HAM is 5000, the name of the license file would be

"5000.LIC". The license file for your G Code Controller is

either located on a distribution diskette, or provided by

Ability Systems via Email.

Upgrading from all Previous Versions

This version requires Indexer LPT version 5.04 or above. From

the Indexer LPT Diagnostic program, determine the version of

Indexer LPT from the sign on screen. If it is below 5.04, you

will need to upgrade Indexer LPT.

Make sure that you have no important information stored in the

sample job SAMPLE.CAM. If you do, you may save this job using

the G Code Controller's File->Save As menu. Save SAMPLE.CAM under

another file name.

ALSO - Make sure that you have saved the sample tool offset file

SAMPTOOL.OFF under another file name using

Edit > Tool Offset Table > Save (Save As).

Upgrading from Version 1.1x

From the control panel, select "Add Remove Programs". Select

"G Code Controller" from the list of programs, then

snap "Add/Remove". Select "Remove all Components".

All components will be removed that were installed from the

original distribution diskette. Other files that you may

have created will be left alone, including the system

setup file GIXW.INI, which resides in the WINDOWS

directory.

This version is supported under the license file that was

provided with previous versions of 1.1x

Install this version of G Code Controller by running the

Setup.EXE program provided on the distribution diskette.

Upgrading from Version 1.0x

Locate the original distribution diskette from your PREVIOUS

version of G Code Controller. Make sure that the write protect tab

is covered (not write protected). Place this diskette in the

floppy disk drive of your computer.

From the Control Panel, double snap Add/Remove Programs, select

"G Code Controller" and snap "Add/Remove". Select "Remove all

Components". (All components will be removed that were installed

from the original distribution diskette. Other files that you

may have created will be left alone). Your PREVIOUS version G Code

Controller will be written to the diskette. As per your Upgrade

Agreement, return the PREVIOUS version diskette to Ability

Systems.

Copy the license file provided by Ability Systems to the

Indexer LPT directory.

Install the NEW version of G Code Controller by running the

Setup.EXE program provided on the NEW distribution diskette.

RELEASE NOTES

Version 1.14

Increased capacity to store long file names and paths to a total

of 512 characters.

Version 1.13

Enhanced list directives by adding a means to save and retrieve

numeric variables within M, T and S code lists. See the section

entitled DIRECTIVES, below under ADDITIONAL FEATURES, for a

descriptions of available list directives.

Version 1.12

The Feed Rate Override feature of Indexer LPT can now be

selectively implemented, and no longer requires enabling (and

disabling) using Indexer LPT's "save_fro_enable" command.

The Feed Rate Override feature is enabled for the system by

means of the Setup->Feed Rate Override dialog. Do NOT enable the

feed rate override feature if the feed rate override

potentiometer is not physically connected to the Hardware Assist

Module, as erratic speeds may result.

Once the Feed Rate Override feature is enabled for the system,

it can be selectively applied to feed rate (G01, G02, G03) or

rapid traverse (G00), or both for any job using the dialog

selected by the Edit->Defaults->Feed Rate Override menu. This

dialog provides for different settings corresponding to

"Production Run" or "Dry Run" operations.

Typically, Feed Rate Override is applied only to feed rate for

"Production Run", since it is most often used to make fine

adjustments in cutting speeds. In this case, therefore, it is

most appropriate to disable the override for rapid traverse,

thereby leaving those rates at their optimum system setting.

Alternately, in jobs where the feed rates have been determined,

it may not be desirable to have them altered from the control

panel. For these jobs, the feature can be disabled for both

rapids and feeds.

During "Dry Run" operations it is often desirable to slow both

feed and rapid traverse, especially when checking for tool path

interference in tight spots.

These settings for Feed Rate Override can be saved within the

CAM file, and thus vary from job to job as the need requires.

When the Feed Rate Override feature is enabled for the system

via the System->Feed Rate Override dialog, the percentage value

of feed rate override, corresponding to the position of the Feed

Rate Override potentiometer, is displayed on the "Production

Run" and "Dry Run" dialogs, even if the feature is selectively

disabled for the job.

Version 1.11

The limit switch seek sequence can now be prioritized, allowing

specific ordering of the stages when multiple stages are selected

for automatic limit switch homing.

A list of Indexer LPT commands can now be automatically executed

when the program loads. The purpose of this list is to allow

certain Indexer LPT initialization sequences. Since this list

executes automatically, and potentially inadvertently, commands

that cause physical motion, or which can result in injury or harm

should be avoided. This list will NOT accept the following Indexer LPT

commands:

feed

move

home

jog

arc_to..

circle

q_go

Version 1.10

This software is no longer protected to the installation

diskette, but requires Indexer LPT version 5 or above.

Version 1.06

Provision for automatic loading from the Startup group: This

accommodates OEM machinery where it is desired that GCC load

automatically when the computer is powered up. To do this, you

can make a shortcut to GIXW.EXE in the Startup group. The

software will start loading automatically, but it will pause if

Indexer LPT is still loading, then complete loading itself when

Indexer LPT is ready.

Provision for multiple path return from tool change position:

Additional macro list directives were added as described below

under ADDITIONAL FEATURES. These features also provide for

assigning a unique position for each tool, instead of requiring

that all tools be picked up and placed in a single tool change

position.

Reserve Enough Look-Ahead Memory!

Make sure that the look-ahead (queue) memory reserved under

Indexer LPT/XQ is sufficiently large. You can do this from the

Indexer LPT Diag. Launch the Indexer LPT Diag by snapping

Start->Programs->Indexer LPT Diag. From the Command Motor

dialog, set the available queue memory to a sufficiently large

number (for example, 8000000 ) using the Indexer LPT "set_q_mem"

command. The best amount for your purposes will depend on the

complexity of the shapes that you anticipate.

The amount of memory reserved is saved to disk, and it will

remain in effect until it is changed again with another

"set_q_mem" command.

Feed Hold

Make sure that the Indexer LPT feed hold feature is installed.

Loading a Sample Job

After you installed the program and completed the System Setup

procedures outlined in the Users Guide, you may wish construct a

sample job. Here is how:

1) From the File menu, snap Open CAM File. Double snap over

Sample.cam

2) From the Edit menu select Programs. Snap the Browse button

associated with the Program File Specification field.

Double snap over:

square sample.txt

3) Snap Ok, then select File->Save as. Rename the File name to:

square.cam

4) Snap Save. Snap Yes to the following pop-up prompts that

may appear.

ADDITIONAL FEATURES

These features may not appear the latest G Code Controller

manual:

Macro processing "directives" accommodate designs with multiple

tool change positions, and designs that require the return from

tool change position over a complex path.

List directives are described in the G Code Controller manual in

the chapter entitled "System Set-Ups" following the explanation

of the dialogs that are used to customize M and T codes.

DIRECTIVES

#save_num(#<index>)(<number>)

This directive saves a floating point number as a variable for

use by a subsequent directive. The <index> that is used to store

(and later retrieve) the variable can be any whole number

preceded by the pound character, "#". The <number> variable can

be any decimal number. For example, if you wanted to save the

number, 12.125, and retrieve it later using an index of 100, the

list directive would look like this:

#save_num(#100)(12.125)

The number saved will persist until G Code Controller is

unloaded, or until it is cleared by a #clear_num or #clear_nums

directive.

The assigned variables and associated values can be viewed from

the dialog invoked by the Diagnostic->Display Machine Variables

menu.

#clear_num(#<index>)

This directive clears the entry for the system variable saved

under #<index>. For example, the list directive to remove

variable saved under index 100 would look like this:

#clear_num(#100)

#clear_nums

This directive clears all of stored variables.

#rapid_to(<Stage>)(<Position>)

#rapid_to(<Stage>)(#<index>)

This directive moves the designated stage to an absolute

Position relative to machine home.

This directive will place the last position in the top of a

return list, which collects the departure positions of up to ten

#rapid_to directives. Motion can be reversed to previous

positions for each stage using subsequent #rapid_return<Stage>

commands. If the return list is full, the last position will be

placed at the top of the list, but the position at the bottom of

the list will be lost.

This directive will accept either an immediate decimal number

argument for <Position> or an #<index> to access a system

variable.

For example, to move the X stage to a position that is 5.432

inches from the machine home position, the directive to

accomplish this using an immediate value for <Position> would

look like this:

#rapid_to(X)(5.432)

Suppose, on the other hand, that the number "5.432" was saved to

the variable index "10" using this directive:

#save_num(#10)(5.432)

Then the X stage could be subsequently moved to that position

with this list directive:

#rapid_to(X)(#10)

If a variable did not exist under the designated index ("#10" in

this example) the #rapid_to directive will be ignored.

#rapid_return(<Stage>)

This directive will retrace retrace to previous stage positions

in the reverse order of the #rapid_to commands, up to ten deep.

Each time #rapid_return() executes, it frees an entry position

in the return list.

#clear_last_return(<Stage>)

This directive removes the last entry from the top of the return

list and frees an entry position in the list. You can use this

directive if (for some reason) you need to issue a #rapid_to

directive without affecting the existing return list. The

#rapid_to directive places the last position at the top of the

list. The #clear_last_return() removes the value from the top of

the list

#clear_all_returns(<Stage>)

This directive clears the entire return list for the designated

<Stage>